# Staff requirement, offshore patrol vessel

SDO/NSR/PATROL-2031   Rev F   RESTRICTED

# 1.1 Mission profile

The platform shall sustain a 21-day patrol without replenishment, with an endurance of 4,500 nautical miles at 14 knots economical speed. Maximum continuous speed shall be not less than 24 knots in sea state 3 at full load displacement. The platform shall operate a 10-tonne class helicopter and shall launch and recover a 7 m rigid hull inflatable boat at speeds up to 10 knots in sea state 4.

# 2.4 Accommodation standard

Design complement is 96 with margin for a further 20 embarked personnel. Junior rates shall be berthed in cabins of not more than six, with a minimum of 2.8 square metres of deck area and 0.85 cubic metres of stowage per person. No accommodation space shall have its only escape route through a machinery space.

# 6.1 Weight margin policy

A design and build margin of 6 percent of lightship weight and a KG margin of 0.25 m shall be carried through to contract design. Service life allowance shall be a further 5 percent of lightship displacement and 0.15 m of KG, reserved against equipment growth over 30 years.